A_Practical_Approach_Dynamical_Systems_Engineers_Patricia_C02

pag17

CHAPTER 2 System Modeling

2.1 INTRODUCTION

What is a system model? At the most basic level, a model of a system describes how that system behaves. Such descriptions can be in the form of words, diagrams, or equations depending on what is appropriate for the system being modeled. Although systems can include people or businesses (think of role models or business models), we will be concerned with physical systems and their mathematical representation.

—-------------------------------------------------------

¿Qué es un modelo de sistema? En el nivel más básico, un modelo de un sistema describe cómo se comporta ese sistema. Dichas descripciones pueden tener la forma de palabras, diagramas o ecuaciones, según lo que sea apropiado para el sistema que se está modelando. Aunque los sistemas pueden incluir personas o negocios (piense en modelos a seguir o modelos de negocios), nos ocuparemos de los sistemas físicos y su representación matemática.

—-------------------------------------------------------

In Chapter 1, we discussed how a system can be represented simply and abstractly by a block diagram such as the one shown in Figure 2.1, in which there is an input to the system and an output from the system. In this chapter, the focus is on what happens inside the block and the mathematical relationship between the input and the output.

—-------------------------------------------------------

En el Capítulo 1, discutimos cómo un sistema puede representarse de manera simple y abstracta mediante un diagrama de bloques como el que se muestra en la Figura 2.1, en el que hay una entrada al sistema y una salida del sistema. En este capítulo, el enfoque está en lo que sucede dentro del bloque y la relación matemática entre la entrada y la salida.

—-------------------------------------------------------

System models can take several forms, and some of these forms depend on the type of system. Every system we present can be modeled using either differential equations (for continuous-time systems) or difference equations (for discrete-time systems). There is also a class of system known as hybrid systems that have both continuous-time and discrete-time characteristics. Physical hybrid systems are typically modeled using differential equations with discrete transitions. A classic example of a hybrid system is a bouncing ball when there are continuous-time dynamics when the ball is in the air and discrete transitions when it hits the ground. In any case, these differential and difference equations are time domain models. In Section 2.2, these equations are discussed in detail. These equations can also be generalized into a standard form known as a state-space model, which is covered in Section 2.4.

—-------------------------------------------------------

Los modelos de sistema pueden tomar varias formas, y algunas de estas formas dependen del tipo de sistema. Cada sistema que presentamos se puede modelar utilizando ecuaciones diferenciales (para sistemas de tiempo continuo) o ecuaciones en diferencias (para sistemas de tiempo discreto). También hay una clase de sistema conocida como sistemas híbridos que tienen características tanto de tiempo continuo como de tiempo discreto. Los sistemas híbridos físicos generalmente se modelan utilizando ecuaciones diferenciales con transiciones discretas. Un ejemplo clásico de un sistema híbrido es una pelota que rebota cuando hay una dinámica de tiempo continuo cuando la pelota está en el aire y transiciones discretas cuando toca el suelo. En cualquier caso, estas ecuaciones diferenciales y en diferencias son modelos en el dominio del tiempo. En la Sección 2.2, estas ecuaciones se discuten en detalle. Estas ecuaciones también se pueden generalizar en una forma estándar conocida como modelo de espacio de estados, que se cubre en la Sección 2.4.

—-------------------------------------------------------

Although differential and difference equations can tell us a lot about a system, sometimes they do not provide information about the system in a form in which we can easily identify certain system characteristics. Another way of representing a linear system model, which provides an alternative perspective, is in the Laplace domain or z domain using transfer functions. In this form, we can more easily see the frequency response of a system and its stability properties. Transfer functions of both continuous-time and discrete-time systems are discussed in Section 2.3.

—-------------------------------------------------------

Aunque las ecuaciones diferenciales y en diferencias pueden decirnos mucho sobre un sistema, a veces no brindan información sobre el sistema en una forma en la que podamos identificar fácilmente ciertas características del sistema. Otra forma de representar un modelo de sistema lineal, que proporciona una perspectiva alternativa, es en el dominio de Laplace o en el dominio z utilizando funciones de transferencia. De esta forma, podemos ver más fácilmente la respuesta de frecuencia de un sistema y sus propiedades de estabilidad. Las funciones de transferencia de los sistemas de tiempo continuo y discreto se analizan en la Sección 2.3.

—-------------------------------------------------------

Finally, in Section 2.5, the determination of model parameters is discussed. System identification refers to the process of figuring out the constants of a system through experimentation. This process is different from modeling a system from physical principles, as can be done with simpler systems.

With system identification, a system is treated more like a black box, and experiments and tests need to be performed to determine the parameters.

It is important to point out that a system’s model is only a representation of its behavior and an imperfect one at that. A mathematical model of a real system can never account for everything, and there will always be some source of error to mess up the description it provides. When finding these imperfections, there is always the temptation to “fix” them by modifying the model to better fit the behavior we see. However, this temptation must be kept at bay to prevent our models from becoming too complex. One has to keep in mind the tradeoff between complexity and accuracy and decide when the model is “good enough” for its purpose. Unfortunately, there is no black and white answer to the question of how good is “good enough.” The answer is application specific and can only be found through experience.

One example of a system that can have different types of models is vehicle traffic. On the one hand, the traffic can be thought of as fluid flow with varying density as the vehicles get bunched or spread apart. In this scenario, fluid dynamics equations can be used for the model. On the other hand, the model may be formulated to keep track of individual vehicles and their interactions with the other vehicles and the road environment. Then in this scenario, equations of motion for each individual vehicle would be used, and their combination would describe the entire system. These two types of models treat vehicle traffic at different scales.

The fluid model is macroscopic, and the individual vehicle models are microscopic. Depending on the application, one level of detail may be more appropriate than the other.

2.2 EQUATIONS OF MOTION

When we refer to equations of motion, we are referring to a set of equations that describes how a system changes in time. For the car suspension system previously discussed, these equations describe the up and down motions of the car relative to the road surface as it drives and experiences variations in the road surface. In this example, there truly is motion in the physical sense. Equations of motion, however, are more general. As another example, consider a bank account earning interest. There is no physical motion happening in the account, but the money is in fact “moving” in the sense that it is changing value with time.

In this section, we develop equations of motion for several different systems (car suspension, car kinematics, bank account, and a computercontrolled vehicle) and show how to use MATLAB to represent and solve these equations.

2.2.1 Differential Equations for Continuous-Time Systems

Let us start with the popular example of the car suspension as shown in Figure 1.2. We will first develop the equations of motion from physical principles and then discuss the form and meaning of these equations.

A simplified version of the system is shown in Figure 2.2. This version consists of two masses. The bottom mass m1 represents the wheel, and it is connected to the ground by a spring (the flexion of the tire). In this simple model, we assume that the tire’s damping is negligible (a simplification that leads to a “good enough” model in this case). The top mass m2 represents the portion of the car body supported by the wheel. The mass m2 is connected to the wheel mass by a spring and damper (the shocks and linkage connecting them together). The variables x1 and x2 denote the height of the wheel and car, respectively, and the arrows indicate the direction of positive values. We assume that x1 ¼ 0 and x2 ¼ 0 are the equilibrium positions of both masses where the springs are not compressed or stretched. We also assume that r is the height of the road above some reference.

To develop the equations of motion for this system, we first draw a free body diagram for each mass and then apply Newton’s second law of motion. The free body diagram for m1 is shown in Figure 2.3. The large arrows indicate the direction of the force being applied to the body caused by the movement of both masses. The dots above x1 and x2 indicate derivatives with respect to time.

According to Newton’s second law, summing the forces on this body and setting it equal to mass times acceleration gives

k2ðx2  x1Þ þ bðx_2  x_1Þ  k1ðx1  rÞ ¼ m1€x1 (2.1)

Note that the effect of gravity can be ignored if we define our displacements, x1 and x2, with respect to the equilibrium positions of the springs. Gravity is taken into account when we define the x1 ¼ 0 and x2 ¼ 0 positions by these equilibria because the gravitational force creates an offset that is eliminated by this definition of zero displacement.

We now repeat the process for m2. First the free body diagram is shown in Figure 2.4

Then summing the forces on m2 gives

k2ðx2  x1Þ  bðx_2  x_1Þ ¼ m2€x2 (2.2)

Finally, the car suspension system dynamics can be represented by the two equations of motion.

€x1ðtÞ þ b m1 x_1ðtÞ þ k1 þ k2 m1 x1ðtÞ  b m1 x_2ðtÞ  k2 m1 x2ðtÞ ¼ k1 m1 rðtÞ €x2ðtÞ þ b m2 x_2ðtÞ þ k2 m2 x2ðtÞ  b m2 x_1ðtÞ  k2 m2 x1ðtÞ ¼ 0

Note that these equations describe a fourth-order system (i.e., there are two states x1 and x2, each with a second derivative). The final form in (2.3) is in monic form. Also note that the variable t has been added to show explicitly which variables depend on time and which do not. However the (t) is often dropped for brevity. The characteristics of monic form are:

1. State variables (x1 and x2) are collected on the left side of the equation.

2. Input variables (r) are collected on the right side of the equation.

3. State variables are collected together and arranged in descending order of the derivatives.

4. The highest derivative term in each equation has a coefficient of 1.

It is useful to put equations in a standard form because it flows easily into the concepts of transfer functions and state-space representations of system. We will see this connection when these concepts are discussed in Sections 2.3 and 2.4.

The model in (2.3) is an ordinary differential equation (ODE) because it has one independent variable (t). In contrast, partial differential equations (PDEs) have more than one independent variable. Partial differential equations are not discussed in this book, but interested readers may consult Farlow (1993) as a starting point

2.2.1.1 MATLAB Example: Car Suspension

We now discuss how to represent and simulate the car suspension system in MATLAB. The basic m-file that runs the simulation is shown below, and the simulation results are shown in Figure 2.5. First we show the m-file that runs the basic simulation and describe how the simulation works.

% suspension_simulation.m % Close all figures and clear all variables close all clear all % How long to simulate (in seconds) t_end ¼ 5; % Set initial conditions on the system x1_0 ¼ 0; x2_0 ¼ 0; x1_dot_0 ¼ 0; x2_dot_0 ¼ 0; % Solve the system equations [T X] ¼ ode45(@suspension_model,[0 t_end],[x1_dot_0 x1_0 x2_dot_0 x2_0]); % Save the results in a vector x1_dot ¼ X(:,1); x1 ¼ X(:,2); x2_dot ¼ X(:,3); x2 ¼ X(:,4); % Plot the results plot(T,x1,'k',T,x2,'k--') xlabel('Time (s)') ylabel('Height (m)') title('Wheel Height and Car Height vs. Time') legend('Wheel Height, x_1','Car Height, x_2')

We now discuss the above code in detail and describe each line.

close all clear all

These first two lines of the code close figure windows and clear variables so that the simulation is starting with a clean slate.

t_end ¼ 5;

Next the simulation time is set and stored in the t_end variable. This value is used later in the code to solve the system equations for the specified amount of time (5 seconds in this case). Note that MATLAB does not assign units. Rather, it is up to the user to interpret the results and associate appropriate units with the values. We may wish to associate any system of units with (2.3), but in this example and throughout the book, we generally use SI units.

x1_0 ¼ 0; x2_0 ¼ 0; x1_dot_0 ¼ 0; x2_dot_0 ¼ 0;

In these four lines, the initial conditions are set for the system. In this simulation, the wheel height, car height, and their derivatives set to zero, meaning the system is initially at rest. These values are used to solve the system equations.

[T X] ¼ ode45(@suspension_model,[0 t_end],[x1_dot_0 x1_0 x2_dot_0 x2_0]);

The next line is the one that solves the system equations given by (2.3).

The command ode45 is MATLAB’s differential equation solver that can be used for nonstiff 1 ordinary differential equation problems with a medium order of accuracy. Several other ODE solvers are available (e.g., ode23 and ode113) that may be more appropriate for certain equations.

The ode45 command uses three parameters to describe how the simulation should run. First,thefile containing thesystem equations (suspension_model.m described below) is included after the @ symbol, indicating a function handle. Second, the simulation time parameters are included in a vector, indicating that the equations should be solvedstarting at time zero and ending at t_end. Finally, the initial conditions are included in a vector as the third parameter in the command.

Last, the ode45 command returns the results of the solution to the vector [T X]. After the solver runs, the time vector is stored in T, and the solution to the differential equations is stored in matrix X. In the case of the car suspension system, T has 437 elements, and X is a 437  4 matrix with each row corresponding to the time and each column corresponding to the states x_1, x1, x_2, and x2 in the order that they appear in the initial conditions and suspension_model.m file.

x1_dot ¼ X(:,1); x1 ¼ X(:,2); x2_dot ¼ X(:,3); x2 ¼ X(:,4);

These four lines extract the values from the X matrix and put them into separate vectors for each state. This step is not necessary, but it simplifies working with the data.

plot(T,x1,'k',T,x2,'k--') xlabel('Time (s)') ylabel('Height (m)') title('Wheel Height and Car Height vs. Time') legend('Wheel Height, x_1','Car Height, x_2')

These last five lines in the file plot x1 and x2 versus time for the 5 s of the simulation, add labels, and format the plot. The results of the plot are seen in Figure 2.5. In the plot, notice that the wheel height and car height are at zero until 1 s, when there is a sudden increase and then they settle to a value of 0.1. The fact that the wheel and car heights are both zero does not mean they are at the same height but rather that they are in their equilibrium position with the springs not stretched or compressed beyond what gravity would do.

The jump in values at 1 second is a result of changing the input r(t) from 0 to 0.1. There was nothing in the main simulation program discussed earlier that set the value of r, so where does this occur? The answer lies in the other file specifying the system model, suspension_model.m shown below.

function dy ¼ suspension_model(t,y) % Define the model parameters m1 ¼ 70; % kg m2 ¼ 350; % kg k1 ¼ 176000; % N/m k2 ¼ 27000; % N/m b ¼ 2500; % Ns/m

% Define the input r(t) t_start ¼ 1; if (t < t_start) r ¼ 0; else r ¼ 0.1; end dy ¼ zeros(4,1); dy(1) ¼ -(b/m1)*y(1) - ((k1þk2)/m1)*y(2) þ (b/m1)*y(3) þ (k2/m1)* y(4) þ (k1/m1)*r; dy(2) ¼ y(1); dy(3) ¼ (b/m2)*y(1) þ (k2/m2)*y(2) - (b/m2)*y(3) - (k2/m2)*y(4); dy(4) ¼ y(3);

We now describe this m-file line-by-line and how it relates to the system model in (2.3).

function dy ¼ suspension_model(t,y)

The first line of the file defines it as a function and gives its name (suspension_model), which is used in the ode45 command. The vectors t and y hold the simulation time parameters [0 t_end] and initial conditions [x1_dot_0 x1_0 x2_dot_0 x2_0], respectively.

m1 ¼ 70; % kg m2 ¼ 350; % kg k1 ¼ 176000; % N/m k2 ¼ 27000; % N/m b ¼ 2500; % Ns/m

These five lines give the values to the constants used in the model.2

t_start ¼ 1; if (t < t_start) r ¼ 0; else r ¼ 0.1; end

The if statement defines the input r(t). In this case, we wish to have r(t) be a delayed step function with a value of 0.1 after 1 second, as shown in Figure 2.6. The transition time is set in the variable t_start, and when the ODE solver time is less than this value, the input is zero. When the time exceeds 1 s, the input is 0.1. These input values are used in the system equations that follow.

dy ¼ zeros(4,1);

This line initializes the size of dy as appropriate. The vector dy stores the derivatives of each state variable, which get modified according to (2.3). In this case, the system is fourth order, and the vector is created with four elements. Initially, the values are set to zero, but these will be overwritten with the initial values of the system when the solver runs.

dy(1) ¼ -(b/m1)*y(1) - ((k1þk2)/m1)*y(2) þ (b/m1)*y(3) þ (k2/m1)* y(4) þ (k1/m1)*r; dy(2) ¼ y(1); dy(3) ¼ (b/m2)*y(1) þ (k2/m2)*y(2) - (b/m2)*y(3) - (k2/m2)*y(4); dy(4) ¼ y(3);

These next four lines specify the system using (2.3), and we will go through a bit of detail to explain where they came from. The system is a fourth-order system and thus needs four equations in the model, but there are only two equations specified in (2.3). Where do the other two equations come from?

The answer lies in the concept states of the system, which is described in more detail in Section 2.4, but we will use it here to our advantage in creating the model equations in MATLAB. Two obvious choices for states are x1 and x2, the wheel height and car height, respectively. We can generate two more states by taking their derivatives, x_1 and x_2. Why not take another derivative to get two more states €x1 and €x2? One reason is that the system is fourth order, and when we took the first derivative, that gave us four states, and no more are needed. Another reason is that taking more derivatives will cause problems in the procedure we are about to outline.

The model equations in the function file need to provide update equations for each of the system’s states with respect to the system states and inputs only. In the case state x1, we need to tell MATLAB how x1 changes with time; this is done by specifying its derivative in terms of the states and inputs. In other words,

x_1 ¼ x_1 (2.4)

This may seem like a trivial equation, but it needs to be interpreted properly. The left side represents the derivative of x1, where x1 is a state. The right side represents the derivative of x1, which is itself a state of the system.

The update equation for the state x_1 is not trivial and a bit more complicated. How does x_1 change with time? We specify it in terms of its derivative,

€x1 ¼  b m1 x_1  k1 þ k2 m1 x1 þ b m1 x_2 þ k2 m1 x2 þ k1 m1 r (2.5)

which is obtained by solving the first equation in (2.3) for €x1. Similarly, we can go through the same process for x2. The update equation for x2 is

x_2 ¼ x_2 (2.6)

and the update equation for x_2 is

€x2 ¼  b m2 x_2  k2 m2 x2 þ b m2 x_1 þ k2 m2 x1 (2.7)

We can now see why €x1 and €x2 cannot be states. The update equations would need to involve €x€1 and €x€2, which do not show up anywhere in the derivation of the model.

For ease of notation, let’s define

y1 ¼ x_1 y2 ¼ x1 y3 ¼ x_2 y4 ¼ x2

Then the update equations become

y_1 ¼  b m1 y1  k1 þ k2 m1 y2 þ b m1 y3 þ k2 m1 y4 þ k1 m1 r y_2 ¼ y1 y_3 ¼  b m2 y3  k2 m2 y4 þ b m2 y1 þ k2 m2 y2 y4 ¼ y3 (2.9)

These final equations are the ones that appear in the model file. Note that the choice of y1, y2, y3, and y4 was arbitrary. We could have equivalently set them to any order of x_1, x1, x_2, and x2. We will see in Section 2.4 that there are conventions about how to define the state variables that make certain forms of model immediately identifiable and easy to study.

The car suspension system is an example of a linear system with one input. The system is described by a set of linear differential equations, meaning that the equations are linear combinations of the states and their derivatives. Furthermore, these differential equations obey the principles of scaling and additivity as discussed in the previous chapter. We contrast this with an example of a nonlinear system and the derivation of its nonlinear differential equations.

Consider a kinematic model for a car. In this model, we assume there is no slip on the wheels, and the vehicle is traveling in the x, y plane with no vertical movement. We also consider only the movement of the vehicle and not acceleration. Under many conditions (e.g., on dry roads and when the car is being driven within its handling limits), these assumptions are valid, and the kinematic model’s behavior closely matches the actual car. Given these assumptions, the mathematical model can be derived starting with the coordinate definitions shown in Figure 2.7.

The exact position and orientation of the car in the global coordinate system can be described by four variables. The (x, y) coordinates give the location of the center of the rear axle. The car’s angle with respect to the x-axis is given by q. The steering wheel’s angle with respect to the car’s longitudinal axis is given by 4. The state of the car can be described by these four variables, and thus it is a fourth-order system requiring four update equations.

From the no-slip constraints, the instantaneous velocity of the car in the x and y directions is given as

x_ ¼ v1 cos q y_ ¼ v1 sin q (2.10)

where v1 is the linear velocity of the rear wheels. The location of the center of the front axle (x1,y1) is given by

x1 ¼ x þ l cos q y1 ¼ y þ l sin q (2.11)

and the velocity of this point is given by

x_1 ¼ x_  l _ q sin q y_1 ¼ y_ þ l _ q cos q (2.12)

Now if we apply the no-slip condition to the front wheels, meaning there can be no velocity component perpendicular to the direction of wheel travel, then

y_1 cosðq þ 4Þ ¼ x_1 sinðq þ 4Þ (2.13)

Substituting x_1 and y_1 from (2.12) into (2.13) and solving for _ q gives

_ q ¼ tan 4 l v1 (2.14)

Then the complete model for the car is

x_ ¼ v1 cos q y_ ¼ v1 sin q _ q ¼ tan 4 l v1 4_ ¼ v2 (2.15)

This is a two-input system in which v1 is the linear velocity of the rear wheels and v2 is the angular velocity of the steering wheels.

2.2.1.2 Simulink Example: Kinematic Car Model

Below is a simulation of the kinematic car model using Simulink. This system could be modeled in MATLAB using the above procedure for the suspension system, but instead we will demonstrate the power of Simulink. The Simulink model for the system is shown in Figure 2.8. The model in the figure is annotated to indicate the location of x, y, q, 4, l, v1, and v2. Creating and running the model in Simulink is easier than writing the MATLAB code and gets the same results. However, the MATLAB code offers much more flexibility when system interactions become more complex.

The results of running the Simulink simulation for 10 s are shown in Figures 2.9 and 2.10. The length of the car is set to 1 m, the velocity input v1 is set to a constant of 5 m/s, and the steering velocity input v2 is set to a sinusoid of amplitude 0.5 rad/s and frequency 1 rad/s. As with the previous MATLAB example, units are not associated the values but must be interpreted by the user. In Figure 2.9, all the states are plotted versus time using “Scope” blocks. In Figure 2.10, the x,y position of the car is shown having been plotted using the “XY graph” block. The initial position of the car is at (0,0) facing along the x-axis and with straight front wheels because the initial conditions of each integrator was set to zero (the default value). Changing the initial position of the car is obtained by setting the integrator initial conditions.

2.2.2 Difference Equations for Discrete-Time Systems

Just as differential equations describe the dynamics of continuous-time systems, difference equations describe the dynamics of discrete-time systems. Although the principle is the same, difference equations are often easier to derive and simulate because of their discrete, step-by-step nature.

As a first example, let’s again consider the bank account with accruing interest as discussed in Chapter 1. In this example, the formula for the amount of money in the account is

An ¼ A0  1 þ r 12 n (2.16)

where r is the annual interest rate, n is the number of months, A0 is the initial deposit, and An is the amount of money in the account in month n. This variable n represents a discrete interval of time. If we think about how the amount of money changes at each step, this will give a difference equation to update A, similar to the role differential equations played in the previous two examples. Mathematically, the change is expressed as

An  An1 ¼ A0  1 þ r 12 n  A0  1 þ r 12 n1 ¼ A0  1 þ r 12 n11 þ r 12   1  ¼ An1  r 12  (2.17)

Solving for An yields an update equation.

An ¼ An1 þ An1  r 12  (2.18)

The equation lends itself nicely to a difference equation form, which looks similar but has some notational differences.

A½n ¼  1 þ r 12  A½n  1 (2.19)

In this form of the equation, the [n] is used to explicitly show how the expression depends on the discrete-time variable n. It is analogous to showing (t) in the continuous-time case. This equation is already in standard form because the current value of A, denoted by A[n], is isolated and has a coefficient of 1. Note that r is a constant (the interest rate), so the coefficient of A[n  1] is a constant. As with the continuous-time system, this standard form allows us to more efficiently convert to transfer functions and state-space form.

2.2.2.1 MATLAB Example: Bank Account with Interest

We now discuss how to represent and simulate the bank account in MATLAB. You will notice that the discrete-time system simulation file is much simpler than the continuous-time case, mainly because the discrete-time update equation is almost exactly in the form that can be used in MATLAB, and each entry in the vector represents a discrete time step. One does need to be careful about indices for the vectors, however. In the example, the 0 subscript refers to the initial time, but MATLAB does not allow 0 as an index to its arrays. Rather, array indices start at 1, which can cause confusion between the convention used in mathematical representations and its MATLAB implementation. For this reason, two separate array variables are created: n to represent the month as in the equation and k to represent the array index. These two arrays are the same length, but the values in them are offset by 1.

The basic m-file that runs the simulation is shown below, and the simulation results are shown in Figure 2.11.

% bank_account.m close all clear all r ¼ 0.05; % interest rate n_max ¼ 11; % how long to run the model n ¼ 0:n_max; % define the n vector A(1) ¼ 10000; % initial deposit for k ¼ 2:length(n) A(k) ¼ (1þr/12)*A(k1); end

plot(n,A,'o') y ¼ [10000:100:10500]; set(gca,'YTick',y) set(gca,'YTickLabel',sprintf('%5.0fj',y)) xlabel('Month') ylabel('Amount')

2.2.3 Models for Hybrid Systems

Now let us consider another example of a real-world discrete-time system: the discretization of a continuous-time system. As discussed in Chapter 1, continuous-time systems are often controlled by some type of computer system (PC based or microprocessor), and these computers are inherently discrete-time systems because they run off a clock and their events occur in discrete time intervals. This is an example of a hybrid system, one that has continuous-time and discrete-time parts. The continuous-time part of the system is the mechanical or physical part such as the turning motor or the vibrating mass. The discrete-time part is the computer control that reads sensors and updates command signals at a specific sampling rate.

Hybrid systems are more general than this computer-controlled example. Simply stated, a hybrid system is one that has discrete and continuous dynamics such as the familiar bouncing ball example. If a ball is dropped from some height, it continues to fall until it hits the ground. Up to this point, the dynamics were continuous, and the ball was falling because of gravity. However, when it hits the ground, at that moment its velocity changes direction. In a perfect collision, it would travel with the same speed in the moments before and after impact but with opposite directions. Its new velocity would then be initial conditions for the system equation.

Another example of a hybrid system is a vehicle with a geared transmission. In each of the gears, the vehicle has certain continuous dynamics that relate the fuel input to the speed and acceleration, and these dynamics depend on what gear is selected. Changing gears is a discrete transition between each of these continuous dynamics.

Hybrid systems are an area of much recent research, and readers are encouraged to explore the references at the end of the chapter (Goebel, Sanfelice, & Tell, 2012; van der Schaft & Schumacher, 2000) for more information.

We illustrate the concept of hybrid systems through an example of a computer-controlled car model. To do this, we will revisit the kinematic car model in Simulink and simulate the system as if the velocity commands were coming from a computer at discrete intervals. But first, an overview of the process is provided.

The system simulation follows the steps shown in Figure 2.12. The first step is to set the initial conditions and initial control inputs to the system. With these initial values known, the system equations are solved for T seconds, where T is the sampling time of the system. After this solution is obtained, we see what the endpoint of the solution is and take it to be the initial condition of the system when solving the equations for the following iteration. Also, the system inputs must be chosen (either by closed-loop feedback or an open-loop scheme). With these new initial conditions and inputs, the system equations are solved for another T seconds. This procedure of determining system evolution in T second intervals continues until enough time has elapsed or a stopping condition is met.

Figure 2.13 shows a possible output of a system simulation. The simulation is carried out over five sample intervals. The curve between each point is the solution to the system equations. Then the endpoint of the curve in each interval is the initial condition to the system equations for the next interval. Because of this condition, the overall solution is continuous, but it may not be differentiable (e.g., at time 2T in Figure 2.13). The reason for this lack of smoothness is the inputs to the system. New inputs are applied at each time T, 2T, 3T, . , and the inputs themselves are piecewise constant, being held at a constant value during the intervals that the system equations are being solved but may change to new values at each sampling time.

2.2.3.1 MATLAB Example: Computer-Controlled Vehicle Dynamics

We now illustrate the above procedure with an example in MATLAB. This simulation is set up in two parts. The MATLAB code simulates what would run on a microprocessor or computer. It reads the current state of the system from sensors and outputs the commands to the system for control. The Simulink block diagram simulates the system dynamics. The MATLAB code calls the Simulink file to initiate the process of solving the system equations.

The MATLAB code is shown below and the Simulink block diagram is shown in Figure 2.14.

% hybrid_car.m close all clear all % Initialize constants T ¼ 0.01; % Sampling time (seconds) t_stop ¼ 10; % Simulation run time (seconds) t ¼ [0:T:t_stop]; % Time vector v1 ¼ 5; % Rear wheel linear speed (m/s) v2 ¼ 0.5*sin(t); % Steering wheel speed (rad/s)

% Set initial conditions x0 ¼ 0; y0 ¼ 0; theta0 ¼ 0; phi0 ¼ 0; % Set up state storage arrays X ¼ zeros(size(t)); Y ¼ zeros(size(t)); THETA ¼ zeros(size(t)); PHI ¼ zeros(size(t)); for k ¼ 1:length(t) % Update state storage arrays X(k) ¼ x0; Y(k) ¼ y0; THETA(k) ¼ theta0; PHI(k) ¼ phi0; % Get current inputs v10 ¼ [0 v1]; v20 ¼ [0 v2(k)]; % Simulate car movement for time T simOut ¼ sim('kinematic_car','StopTime',num2str(T)); % Store last value of state from simulation x0 ¼ simOut.get('x').data(end); y0 ¼ simOut.get('y').data(end); theta0 ¼ simOut.get('theta').data(end); phi0 ¼ simOut.get('phi').data(end); end plot(t,X) title('x vs. t') xlabel('Time (seconds)') ylabel('x (meters)') figure plot(t,Y) title('y vs. t') xlabel('Time (seconds)') ylabel('y (meters)') figure plot(t,THETA)

title('ytheta vs. t') xlabel('Time (seconds)') ylabel('ytheta (radians)') figure plot(t,PHI) title('yphi vs. t') xlabel('Time (seconds)') ylabel('yphi (radians)') For the first part of the code: T ¼ 0.01; % Sampling time (seconds) t_stop ¼ 10; % Simulation run time (seconds) t ¼ [0:T:t_stop]; % Time vector v1 ¼ 5; % Rear wheel linear speed (m/s) v2 ¼ 0.5*sin(t); % Steering wheel speed (rad/s)

We set the sampling time to 0.01 s, set the end time of the simulation to be 10 s, and define the time vector to go from 0 to 10 s in 0.01-s increments. Also, the inputs are chosen for the entire simulation. The rear wheel speed is set to a constant of 5 m/s, and the steering wheel speed changes according to a slowly varying sinusoid. Using this method, the system inputs are known a priori and thus follow an open loop scheme.

% Set initial conditions x0 ¼ 0; y0 ¼ 0; theta0 ¼ 0; phi0 ¼ 0;

Next, the initial conditions are chosen for the system. As in the previous Simulink example, the initial position of the car is at (0,0) facing along the x-axis and with straight front wheels. These variables will be updated at each sampling interval to hold the new initial conditions for the next iteration of the simulation loop.

% Set up state storage arrays X ¼ zeros(size(t)); Y ¼ zeros(size(t)); THETA ¼ zeros(size(t)); PHI ¼ zeros(size(t));

These next four lines are for housekeeping purposes. The variables X, Y, THETA, and PHI are created to hold the values of the car’s four states at each sampling interval so the results can be plotted at the end of the simulation versus time. For this reason, they are made to have the same size as the t vector. The entire array is created before the for loop and set to zero so that MATLAB does not need to resize the array each time through the loop. (This can be a time-saving measure in many situations.)

Now the main for loop begins. First, inside the loop

% Update state storage arrays X(k) ¼ x0; Y(k) ¼ y0; THETA(k) ¼ theta0; PHI(k) ¼ phi0;

These four lines update the storage arrays with the current initial condition. (The first time through the loop, these are set to the user’s initial condition of all zeros.)

% Get current inputs v10 ¼ [0 v1]; v20 ¼ [0 v2(k)];

These following two lines set the inputs for the system equations to be solved in the next line. For the first input, the assignment is redundant because v1 is a constant (5 m/s), and the same value is stored each time through the loop. However, placing the assignment here allows for easy updating of the code for a time-varying v1. The variable v2 is time-varying, and thus the input to the system equation needs to be updated each time through the loop as denoted using the index k, the counter in the for loop.

In both cases, notice the syntax of the assignment statement. The variables v10 and v20 will be passed to the sim command in the next line and will be read into the Simulink model through the “From Workspace” block. This block requires an array containing all the inputs to be applied to the Simulink model and a time index for each one. For this simulation, we are sending a single input value during each iteration; thus, the array has only one row. The zero in the first column is the time index.

% Simulate car movement for time T simOut ¼ sim('kinematic_car','StopTime',num2str(T));

The sim command runs the Simulink model contained in the file “kinematic_car.mdl.” The command allows the user to specify several parameters in the model, but in this case, we only specify the stop time. The “StopTime” parameter is the one we wish to define, and the following argument in the list, num2str(T), is used because the command expects the stop time value to be passed as a string. The entire list of parameters can be found in the MATLAB documentation.

The Simulink model called in this command is shown in Figure 2.14. This model is almost identical to the one used in Simulink Example 2.2.1.2, but there are a few key differences. First is the way v1 and v2 are generated. Here, they are passed from the MATLAB code as described above using the “From Workspace” block rather than created in Simulink using the “Constant” and “Sine Wave” blocks as in the previous example.

Second, the system states x, y, q, and 4 are returned to the MATLAB program using the “To Workspace” blocks. The labels on the blocks must match the variables that are used in MATLAB. In the previous example, these states were simply plotted using the “Scope” blocks. If the plotting was done within Simulink, then only a portion of the entire trajectory would be shown (for that sampling interval) and overwritten the next time the sim command is called.

Finally, the initial conditions of each integrator (the initial conditions of the system) are passed from MATLAB. This is accomplished by double clicking on the “Integrator” block and setting the initial condition to be the variable in MATLAB. The parameters for the x integrator block are shown in Figure 2.15. The name used as the “Initial condition” parameter (x0) must match the variable name in MATLAB. This is the key to allowing us to update the initial condition of the system for each iteration by saving the last values of x, y, q, and 4 and using them as the new initial conditions in the next iteration.

Returning to the sim command in MATLAB, the output stored in the variable simOut is an object containing all the values from the Simulink model simulation. To obtain the values from the simulation, the following object oriented assignment statements are used.

% Store last value of state from simulation x0 ¼ simOut.get('x').data(end); y0 ¼ simOut.get('y').data(end); theta0 ¼ simOut.get('theta').data(end); phi0 ¼ simOut.get('phi').data(end);

In these lines, the .get allows us to access the variables that were sent back from Simulink using the “From Workspace” block. The variable names from those blocks must appear in the parentheses surrounded by single quotes. The .data returns all the values stored in those variables. In our case, we are only interested in the last value of the solution because this represents the value of the variable at the sampling time and is used as the initial condition for the next iteration. Using end as the index of the array allows us to quickly obtain the last value in the array without knowing the exact size of the array.

The final lines of code outside the for loop are used to plot the simulation data and format the figures. The results of the simulation are shown in Figure 2.16. Notice that these results match those obtained in the Simulink simulation of the kinematic car shown in Figure 2.9. The reason for the similarity is the hybrid simulation was run with a sampling time T of 0.01 s, a short interval that accurately models the physical system. In Figure 2.17, the same results are shown with the sampling time increased to 0.5 s. In this case, the results differ and the piecewise nature of the system can be seen.

2.2.4 Flows, Vector Fields, and the Phase Plane

Up to this point, we have looked at the time evolution of the variables in a particular waydplotted with time on the horizontal axis and the variable on the vertical axis. This form of viewing data is intuitive and provides a nice picture of how the individual variables evolve. However, there is another way to look at the data, particularly for second-order systems, which shows relationships between the variables. Known as a phase plot, this is a plot commonly showing x_ versus x, where x_ and x are the states of the system. Time is not shown explicitly, but the resulting curves are the system’s trajectory parameterized by time. We saw a hint of what a phase plot is in Figure 2.10, which shows the (x,y) position of the car as it evolved over time.

In general, an Nth order continuous dynamical system can be represented by a system of N differential equations.

x_1 ¼ f1ðx1; x2; /; xN Þ x_2 ¼ f2ðx1; x2; /; xN Þ « x_N ¼ fN ðx1; x2;/; xN Þ (2.20)

For now, we assume the system inputs are set to zero. We can think of the variables x1, x2, ., xN as flowing through their N-dimensional space, starting at some initial state and moving according to their derivatives. This flow is defined by the vector field (f1, f2, ., fN) which is used to generate the phase plot as shown in Figure 2.18 for a second-order system. In the figure, one of the states is plotted against the other (x2 vs. x1, commonly x2 ¼ x_1). The

phase plot consists of a smooth curve showing how x1 and x2 evolve over time from initial conditions. Each point on the curve has a vector tangent to the curve whose magnitude and direction are defined by ðx_1; x_2Þ, the vector field. If you imagine traveling along the curve, these tangent vectors indicate your velocity. The longer the tangent vector is, the faster you are moving.

Let us illustrate these concepts with a concrete example of a pendulum as shown in Figure 2.19. Assume the pendulum has length l with all its mass m concentrated at the end and that it makes an angle of q with vertical. We now derive the differential equation for this system.

Assume the only downward force on the pendulum is due to gravity, resulting in a torque pulling the pendulum toward q ¼ 0. Also assume there is frictional torque acting in the direction opposite the motion and that it is proportional to the pendulum’s speed. Summing the torques gives

mgl sin q  b_ q ¼ I€q (2.21)

where m is the mass of the pendulum, g is gravitational acceleration, l is the pendulum length, b is the coefficient of friction, and I is the mass moment of inertia. Using I ¼ ml2 and simplifying results in the second-order equation for the pendulum.

€q ¼  g l sin q  b ml2 _ q (2.22)

We next generate a phase plot of _ q vs. q for this system using MATLAB, but first we convert (2.22) into state-space form as we did with the car suspension in MATLAB Example 2.2.1.1.

First define two states

x1 ¼ q x2 ¼ _ q (2.23)

and then obtain the update equations for each state, x_1 and x_2 x_1 ¼ x2 x_2 ¼  g l sin x1  b ml2 x2 (2.24)

2.2.4.1 MATLAB Example: Phase Plot of a Pendulum

We now enter the model of the pendulum into MATLAB much like we did for the previous examples. The simulation results are shown in Figures 2.20 and 2.21 for b ¼ 0 and Figures 2.22 and 2.23 for b ¼ 0.1. The basic m-file that runs the simulation is shown after the figures. This m-file has some differences from the previous examples, and we will discuss those differences.

First let’s look at the results in the figures. Figure 2.20 shows the phase plot for the pendulum with no friction (b ¼ 0). The initial angle is 45 degrees or 0.79 radians, and the initial speed is zero. The initial condition is denoted by the circle on the plot. At this point, there is a vector pointing down. This vector is defined by ð_ q; €qÞ, so the component of the vector along the horizontal axis of the plot is 0 because by the initial condition _ q ¼ 0. The component along the vertical axis is negative, indicating that the pendulum is accelerating back toward q ¼ 0 due to gravity.

At the point (0,-2.4) on the plot, notice that the pendulum is at its vertical position, and its rotational velocity is at its minimum value (although its absolute value is at a maximum). The vector is pointing to the left, indicating that the velocity is negative and its acceleration is zero. Notice also that all around the curve, the vectors are the same length, meaning that ffiffiffiffiffiffiffiffiffiffiffiffiffiffi _ q 2 þ €q 2 q is constant.

Figure 2.21 shows q and _ q versus time. For each time instant, if the values of q and _ q are plotted as points, the phase plot is generated. Furthermore, the slopes of each of the time plots give the horizontal and vertical components of the vector in the phase plot.

Figures 2.22 and 2.23 show the results when there is friction by setting b ¼ 0.1. Starting with the same initial conditions as before, the phase plot (see Figure 2.22) spirals inward toward (0,0). Note that the lengths of the vectors decrease as the curve moves away from the initial point. Eventually, the pendulum stabilizes to q ¼ 0 and stops moving. The origin is an example of a stable equilibrium point (discussed further in Sections 3.3 and 3.4). In Figure 2.23, the angular position and velocity versus time are seen to have oscillations that eventually dampen to zero.

Now let’s discuss how these results were generated. Note that inMATLAB, there are usually several ways to accomplish a desired result, and which one you choose depends on exactly what you want. For example, in this case, the phase plot could have been generated using the odephas2 as the OutputFcn setting in the odeset function. However, this option does not produce a desirable looking plot, so the quiver function was used instead. This way required more code, but the resulting plots were more appropriate for this discussion.

% pendulum_simulation.m % Close all figures and clear all variables close all clear all % How long to simulate (in seconds) t_end ¼ 10; t_sample ¼ 0.1; t ¼ [0:t_sample:t_end]; % Define the model parameters m ¼ 0.1; % kg l ¼ 1; % m b ¼ 0.1; % kg*m ^ 2/s g ¼ 9.8; % m/s ^ 2

% Set initial conditions on the system theta_0 ¼ 45*pi/180; theta_dot_0 ¼ 0; % Create vectors for storing data theta ¼ zeros(size(t)); theta_dot ¼ zeros(size(t)); % Solve the system equations for k ¼ 1:length(t) theta(k) ¼ theta_0; theta_dot(k) ¼ theta_dot_0; [T result] ¼ ode45(@pendulum_model,[0 t_sample],[theta_0 theta_dot_0],odeset,m,l,b,g); theta_0 ¼ result(end,1); theta_dot_0 ¼ result(end,2); end x ¼ theta; y ¼ theta_dot; u ¼ theta_dot; v ¼ -g*sin(theta)/l-b/(m*l^ 2)*theta_dot; % Create the phase plot quiver(x,y,u,v) hold on plot(theta,theta_dot,'b') plot(theta(1),theta_dot(1),'bo') axis([-1 1 -2.5 2.5]) xlabel('ytheta') ylabel('ythetayprime') title('Pendulum Phase Plot') % Plot the results figure plot(t,theta,'k',t,theta_dot,'k–') xlabel('Time (s)') ylabel('Angle (radians) and Angular Velocity (radians/s)') title('Pendulum Angle and Velocity vs. Time') legend('Pendulum Angle,ytheta','Pendulum Velocity,ytheta^yprime')

The above code follows the same basic structure as the hybrid system in the previous example, which simulated the system using sampling. The reason for using the same structure here is that we are “sampling” the trajectory at certain points so that we can plot the vectors at appropriate points along the curve (but not too many to crowd the figure) while still having a smooth curve underneath.

% How long to simulate (in seconds) t_end ¼ 10; t_sample ¼ 0.1; t ¼ [0:t_sample:t_end];

The code begins with initializing the time vector. The simulation is set to run for 10 s with a sampling time of 0.1 s. As in the hybrid system example, the “continuous” part of the system will be executed for 0.1 s to determine what the values of the system variables are during that time. The ending value of these variables are then used as initial condition for the next sampling period, with these values also being stored in an array for plotting the results later in the code.

% Define the model parameters m ¼ 0.1; % kg l ¼ 1; % m b ¼ 0.1; % kg*m ^ 2/s g ¼ 9.8; % m/s ^2

Next, the constants (mass, pendulum length, friction coefficient, and gravitational acceleration) for the problem are defined. This is a change from what was done in previous examples, in which the constants were defined in the model file or Simulink file. The reason for this change is the values must be used to generate the vector field values later in the file. This change also means the ode45 command must be called in a slightly different way, as we will see below.

% Set initial conditions on the system theta_0 ¼ 45*pi/180; theta_dot_0 ¼ 0;

In these two lines, the initial conditions of the pendulum are set to an angle of 45 degrees and an angular velocity of zero.

% Create vectors for storing data theta ¼ zeros(size(t)); theta_dot ¼ zeros(size(t));

The vectors for storing q and _ q are created next. This step is to minimize code run time as before.

% Solve the system equations for k ¼ 1:length(t) theta(k) ¼ theta_0; theta_dot(k) ¼ theta_dot_0; [T result] ¼ ode45(@pendulum_model,[0 t_sample],[theta_0 theta_dot_0],odeset,m,l,b,g); theta_0 ¼ result(end,1); theta_dot_0 ¼ result(end,2); end

This for loop is where the system equations are solved, simulating the “continuous” part of the system as before. There is one difference between this code and the previous examples, and it is how the ode45 command is called. There are extra entries in the parameter list. The odeset function is included to specify that default parameters are used. Then the last set of parameters includes the constants for the system (m, l, b, and g). The pendulum_model.m file is shown after the discussion of the main code.

x ¼ theta; y ¼ theta_dot; u ¼ theta_dot; v ¼ -g*sin(theta)/l-b/(m*l ^ 2)*theta_dot;

In these four lines, the variables needed for the quiver command to generate the phase plot are created. The variable names x, y, u, and v were chosen to match the documentation for the command. The values in x and y are plotted to create the trajectory curve. The values in u and v are the vector fields from (2.24) and are used to plot the arrows along the curve. Because of the way the state variables were defined in (2.23), y and u happen to be the same.

% Create the phase plot quiver(x,y,u,v) hold on plot(theta,theta_dot,'b') plot(theta(1),theta_dot(1),'bo') axis([-1 1 -2.5 2.5]) xlabel('ytheta') ylabel('ythetayprime') title('Pendulum Phase Plot')

Next, the quiver command plots the points with their tangent vectors attached as arrows. There are several options for specifying how the plot looks, including a scaling factor to adjust the lengths of the vector arrows. Following the quiver plot, the full trajectory is plotted in blue (“b”) underneath the arrows, and the initial condition is plotted as “o” in blue as indicated by the “bo” parameter in the plot command. Then the axes are scaled to appropriate values to show the whole plot and labeled. The model file is shown below.

function dx ¼ pendulum_model(t,x,m,l,b,g) dx ¼ zeros(2,1); dx(1) ¼ x(2); dx(2) ¼ (-g/l)*sin(x(1))-(b/(m*l^ 2))*x(2);

This m-file model is similar to the one used in the car suspension example, with one major difference. The constants for the problem are passed to the function as parameters rather than defined in the file so that they can be used in the main m-file.

We have only introduced what phase plots are and how to produce them in MATLAB. However, these plots are rich with information about system behavior. In Chapter 3, we will see how they can be useful tools to study stability and equilibrium points in dynamical systems.

2.3 TRANSFER FUNCTIONS

2.3.1 Overview

So far we have dealt exclusively with systems in the time domain. We have used differential equations and difference equations to mathematically represent how a system behaves, and we have plotted variables versus time and generated phase plots. However, there is another way to mathematically represent systems that is a bit more abstract but holds much information.

A transfer function (or system function) is a frequency domain representation of a dynamical system. Before giving going further, let us first express three assumptions that we will use when discussing transfer functions.

1. Transfer functions are used for linear time-invariant systems. Nonlinear or time-varying systems need different analysis techniques.

2. Transfer functions assume the system is initially at rest (zero initial conditions). An example of trying to use transfer functions with nonzero initial conditions (and the associated difficulties) will be given.

3. Transfer functions describe behavior between a single input and a single

output. Multi-input and multi-output systems have more than one

transfer function to describe the various inputeoutput relationships.

Simply stated, a transfer function of a continuous-time system is

defined by

H.s. . Y.s.

X.s. (2.25)

where X(s) and Y(s) are the Laplace transforms of the system input x(t) and

output y(t) respectively.

For a discrete-time systems, the transfer function is defined by

H.z. . Y.z.

X.z. (2.26)

where X(z) and Y(z) are the z-transforms of the system input x[n] and

output y[n], respectively.

We will not discuss Laplace and z-transform theory in detail, nor will

we derive many of the relationships and characteristics. In the following

sections, we will discuss what the transforms and transfer functions are, how

they are used, and apply them to various examples.

2.3.2 Laplace Transforms for Continuous-Time Systems

Laplace transforms are typically introduced in a course on differential

equations, and they are used to provide an alternative method for solving

differential equations using algebraic methods. This method is quite useful

in dynamical system theory in which systems are described by differential

equations. If we know the input to the system x(t) and the transfer function

H(s), we can determine the output y(t) according to the path shown in

Figure 2.24. In the figure, if x(t) is known, we take the Laplace transform,

denoted by L . Then we multiply the result by H(s) and take the inverse

Laplace transform, denoted by L -1, to get y(t). Although more steps are

required than directly solving the differential equation, the steps are simpler,

and the many properties of Laplace transforms may be used to make the

problem even simpler.

The definition of a Laplace transform of a continuous-time signal x(t) is

X.s. .

ZN

N

x.t.e stdt (2.27)

In this definition, s is the Laplace variable and is a complex number

typically denoted by s . s . ju, where j is

ffiffiffiffiffiffiffi

1

p

.

The inverse Laplace transform is given by

x.t. . 1

2pj

Zs.jN

s jN

X.s.estds (2.28)

In practice, the definitions in (2.27) and (2.28) are rarely used. Instead,

tables of transforms and their properties are commonly used to simplify the

solution procedure outlined in Figure 2.24.

For our purposes in finding the transfer functions of systems, we

introduce notation and properties that will be useful to us. First is to define

the notation for signals and their transforms. Table 2.1 shows three variables

{x(t)}

–1 {x(t)}

X(s)

y(t) Y(s)

x(t)

X(s)H(s)

Laplace

transform

method

Figure 2.24 Steps in the Laplace transform method of solving differential equations.

Table 2.1 Notation for Laplace Transforms

Time Domain

Signal

Laplace

Transform

x.t.4X.s.

y.t.4Y.s.

q.t.4Q.s.

System Modeling 57

we have used thus far, x(t), y(t), and q(t). These time domain signals are

represented by lower case letters and are shown explicitly to be functions of

t. Their Laplace transforms are represented by the same letter capitalized and

are functions of s.

Three properties of Laplace transforms will be particularly useful to us as

we derive transfer functions: linearity and differentiation. These properties

are illustrated in Table 2.2.

As an example of deriving a transfer function, we return to the suspension

system model Equations (2.3) repeated here.

€x

1.t. . b

m1

_x

1.t. . k1 . k2

m1

x1.t.   b

m1

_x

2.t.   k2

m1

x2.t. . k1

m1

r.t.

€x

2.t. . b

m2

_x

2.t. . k2

m2

x2.t.   b

m2

_x

1.t.   k2

m2

x1.t. . 0

(2.29)

We wish to find the transfer function of the system from road input R(s)

to the car height output X2(s). In other words, the transfer function is X2.s.

R.s. .

The first step is to convert (2.29) by taking the Laplace transform term

by term using the properties shown in Tables 2.1 and 2.2. Because the

Laplace transform is a linear operation, the constant multipliers remain

constant.

s2X1.s. . b

m1

sX1.s. . k1 . k2

m1

X1.s.   b

m1

sX2.s.   k2

m1

X2.s. . k1

m1

R.s.

s2 . b

m1

s . k1 . k2

m1

X1.s.

b

m1

s . k2

m1

X2.s. . k1

m1

R.s.

(2.30)

s2X2.s. . b

m2

sX2.s. . k2

m2

X2.s.   b

m2

sX1.s.   k2

m2

X1.s. . 0 (2.31)

Solving (2.31) for X1

X1.s. .

s2 . b

m2

s . k2

m2

b

m2

s . k2

m2

!

X2.s. (2.32)

and plugging into (2.30) eliminates X1, yielding an expression with only X2

as the output and R as the input.

s2 . b

m1

s . k1 . k2

m1

s2 . b

m2

s . k2

m2

b

m2

s . k2

m2

!

X2.s.

b

m1

s . k2

m1

X2.s. . k1

m1

R.s.

(2.33)

Simplifying gives the transfer function:

X2.s.

R.s.

.

k1b

m1m2

s . k2

b

s4 .

b

m1

. b

m2

s3 .

k2

m2

. k1.k2

m1

s2 . bk1

m1m2

s . k1k2

m1m2

(2.34)

This transfer has been put into monic form, meaning the coefficients of

the highest order of s in the numerator and denominator are 1. This is

accomplished by factoring out the

k1b

m1m2

, so it is a multiplier in front of

the fraction.

2.3.2.1 MATLAB Example: Transfer Function for the Car Suspension

MATLAB is well equipped to handle systems described by transfer functions.

Many of the commands we will use here are part of the Control

Systems Toolbox.

To represent the car suspension transfer function, we use (2.34) as

shown in the code below.

% suspension_system_tf.m

close all

clear all

% Define the model parameters

m1 . 70; % kg

m2 . 350; % kg

k1 . 176000; % N/m

k2 . 27000; % N/m

b . 2500; % Ns/m

% Define the transfer function

num . (k1*b/(m1*m2))*[1 k2/b];

den . [1, (b/m1.b/m2), (k2/m2.(k1.k2)/m1), b*k1/(m1*m2), k1*k2/

(m1*m2)];

sys_tf . tf(num,den)

% Plot the step response

step(sys_tf)

This code shows the basics of how to represent the system. First the

constants are defined using the same values as in MATLAB Example 2.2.1.1.

After the constants are defined, the system is created using the tf

command. To use this command, we define the numerator and denominator

of the transfer function according to the coefficients of the polynomials

in s and store them in two different variables, num and den. These

coefficients are simply stored in a row vector and separated by commas

(separation by a space would also work). Notice that the num variable has a

multiplier in front because of the form of (2.34). After the numerator and

denominator are defined, these variables are used as parameters in the tf

command. Because this line does not end with a semicolon, the result is

returned to the command window as shown below.

sys_tf .

1.796e04 s . 1.94e05

———————————————————————————————————————————————

s^4 . 42.86 s^3 . 2977 s^2 . 1.796e04 s . 1.94e05

Continuous-time transfer function.

Checking the result is always a good idea to make sure the coefficients

were entered correctly. One common source of error is to leave out

coefficients. For example, s2. 3 needs to be entered as [1, 0, 3].

The step command produces a plot showing the unit step response for the

system. The default for this command is to set the systeminput to a unit step as

shown in Figure 2.25.The resulting systemoutput plot is shown in Figure 2.26.

The dotted line in the figure indicates the steady-state value for the output.

This step response is almost identical to the one in MATLAB Example

2.2.1.1 except for the scaling and time delay. In the previous example, we set

the input r(t) to a step of height 0.1, not 1, and it transitioned from 0 to 0.1

after 1 s. If we want to replicate that input, we can use the lsim command to

plot the system response to arbitrary inputs. To do this, we define a delayed

unit step input and then use it as a parameter in the command as follows.

T . 0.01;

t_end . 3.5;

r_start . 1;

scale_factor . 0.1;

t . [0:T:t_end];

r . [zeros(1,1.r_start/T),scale_factor*ones(1,(t_end-r_start)/T)];

lsim(sys_tf,r,t)

In the above code, the input r is defined from 0 to 3.5 s with the

transition occurring at 1 s. In the lsim command, the transfer function,

input, and time vector must be specified. The resulting plot is shown in

Figure 2.27. The input is added to the plot by default as a light gray line.

There is an alternative, and possibly simpler, way to create a transfer

function in MATLAB shown below.

% Define the transfer function

s . tf('s');

sys_tf . (k1*b/(m1*m2))*(s . k2/b)/(s^4 . (b/m1.b/m2)*s^3 .

(k2/m2.(k1.k2)/m1)*s^2 . b*k1/(m1*m2)*s . k1*k2/(m1*m2))

Using this method, one enters the transfer function directly as shown in

(2.34) rather than as coefficients in the numerator and denominator. It is

possibly simpler because it is just like the transfer function expression, but it

can be much longer, and the chances are greater of making a mistake while

typing it into the code. Both methods produce the same result and it’s the

user’s discretion to choose the more suitable one.

As a second example, we give a simple model of the human balance

system as a single link inverted pendulum.3 This system incorporates

feedback and we will show how MATLAB can help with the transfer

function derivation.

The single link inverted pendulum is shown in Figure 2.28. In the

figure, the person is represented by a single pendulum rotating about the

ankles. There are three angles associated with the system: qBS, the body’s

angle in space; qFS, the foot’s angle in space; and qBF, the foot’s angle

with respect to the body. Additionally, the person may apply torque T

about the ankle to control balance. For this simple model, we assume the

mass m is concentrated at the center of mass located distance l above

the feet.

The differential equation describing the motion of the system is

mgl sin qBS . T . J€qBS (2.35)

Because this is nonlinear system, it does not have a transfer function

representation. However, using the linear approximation sin qBS z qBS,

the transfer function from T to qBS is found to be4

QBS.s.

T.s.

.

1J

s2 . mgl

J

(2.36)

This transfer function describes how torque applied to the ankle affects the

body angle. To complete the model, we have to account for how a person

might react to changing foot angle qFS. To accomplish this effect, the systemis

modeled using feedback as shown in Figure 2.29. This model assumes only

proprioception is used for feedback (i.e., joint angles are used to determine

body position with respect to itself) and ignores the vestibular and vision systems.

The input to the system is the foot’s angle in space, which represents the

slope on which the person is standing. The output is the body’s angle in space,

which is controlled by applying torque to the ankle. The difference between

these two angles is qBF, which is sensed using proprioception.

Researchers use this model to help understand the human balance

system. One of the key features that they had to identify was the form of

the controller. They have found that a proportional-integral (PI) controller

works when fitting the model to the data. A PI controller has a transfer

function C(s) given by

C.s. . kP . kI

s

(2.37)

where kP and kI are called the proportional gain and integral gain, respectively.

These are constants that must be chosen to match the model to the data.

Also note the left block in the diagramwhose transfer function is e std. This

block represents a delay time of td in the systemcaused by the person’s reaction

time. As discussed earlier, a signal delayed by td in the time domain corresponds

tomultiplication by e std when using Laplace transforms.Researchersmust also

determine td to make the model accurately fit their collected data.

Using properties of feedback loops, the overall transfer function of the

system from qFS to qBS is found to be

QBS

QFS

.

e std

kP . kI

s

1J

s2 . mgl

J

1 . e std

kP . kI

s

1J

s2 . mgl

J

(2.38)

2.3.2.2 MATLAB Example: Transfer Function

of the Human Balance System

We can use MATLAB to represent and simulate the human balance system

shown in Figure 2.29. In this example, we use a new tool to help with

some of the work.

So far we have seen two ways to enter transfer functions. One way is to

enter the transfer function as a numerator and denominator. In this case, it

would mean simplifying (2.38) to a simple fraction with both the numerator

and denominator expressed as polynomials of s and creating two vectors in

MATLAB to hold the polynomial coefficients. However, these coefficients

cannot be found because of the delay component e std . The other way is to

define s using the tf command and then enter the actual polynomials and

let MATLAB do the fraction simplification. In this case, we need to do less

work than the first method because we are required to know that the

feedback system is of the form in (2.38) but do not need to do calculations.

There is a third way to enter the transfer function, and this method puts

most of the work onto MATLAB. The feedback command lets us enter the

individual blocks, and then it does the simplification. The format of the

command is

feedback(M1,M2)

where M1 and M2 show up in the system as shown in Figure 2.30. If not

specified, it is assumed that there is negative feedback.

One other item to discuss about the model is the delay component

e std . Handling this block is easy when we first define s using tf. We

simply use the MATLAB command exp to enter the mathematical

expression.

The code to define and simulate the system is shown below.

% human_balance.m

close all

clear all

% Define the system constants

td . 0.01; % s

J . 81.3; % kg m ^2

m . 82; % kg

g . 9.8; % m/s^2

L . 0.996; % m

kp . 0.5;

ki . 0.001;

% Define the system blocks

s . tf('s');

D . exp(-s*td);

C . kp . ki/s;

G . (1/J)/(s^2.m*g*L/J);

sys . feedback(D*C*G,1)

In the code, first the constants of the system are defined. Some typical

values are used here5 Next the individual blocks are defined using s as

the variable, which allows us to type in the mathematical expressions

rather than just polynomial coefficients. Finally, the system is defined

using the feedback command with the series connections of the three

blocks D, C, and G in place of M1. The result of this command is shown

below.

sys .

a .

x1 x2 x3

x1 0 -2.463 -3.075e-06

x2 4 0 0

x3 0 1 0

b .

u1

x1 0.03125

x2 0

x3 0

c .

x1 x2 x3

y1 0 0.0492 9.84e-05

d .

u1

y1 0

(values computed with all internal delays set to zero)

Internal delays (seconds): 0.01

Continuous-time state space model.

Notice that MATLAB gives us something that looks different from

what we have seen in previous examples. Rather than return a transfer

function, it represents the system with four matrices a, b, c, d. This is

a form of state-space representation, and we will discuss this topic in

Section 2.4.

2.3.2.3 Systems with Nonzero Initial Conditions

Consider the simple circuit shown in Figure 2.31. Assume the capacitor is

initially charged to some nonzero voltage vc(0).

Applying Ohm’s law to the resistor gives

i.t. . vin.t.   vc.t.

R

(2.39)

and taking the Laplace transform of (2.39) results in

I.s. . Vin.s.   Vc.s.

R

(2.40)

Furthermore, the relationship between current and voltage in a

capacitor is

i.t. . C

dvc.t.

dt

(2.41)

Taking the Laplace transform of (2.41) results in the initial condition on

the capacitor appearing

I.s. . C.sVc.s.   vc.0.. (2.42)

Trying to obtain the transfer function Vc.s.

Vin.s. is hopeless. Substituting

(2.42) into (2.40) and simplifying yields

Vc.s. .

1

RC

s . 1

RC

Vin.s.   1

s . 1

RC

vc.0. (2.43)

and shows that the output Vc is a function of both the input Vin and the

initial condition vc(0). However, if we assume zero initial conditions, the

transfer function becomes

Vc.s.

Vin.s.

.

1

RC

s . 1

RC

(2.44)

What if you want to plot the time response of the RC circuit in

MATLAB for nonzero initial conditions? Fortunately, there is a simple

means to accomplish this using the system’s state-space realization. In

Section 2.4, we will show how to carry this out in MATLAB and give a

detailed discussion of state-space models.

2.3.3 z-Transforms for Discrete-Time Systems

The concepts of z-transforms are generally less exposed to mechanical

engineers than they are to electrical engineers, who get a heavy dose of them

in required undergraduate courses in signals and systems theory, which are

then followed by courses in signal processing, communications, and control

systems. In particular, digital signal processing (DSP), digital communication

systems, and digital control systems rely heavily on z-transforms. Mechanical

engineers are less likely to study z-transforms in their undergraduate coursework,

but may encounter them in a course on digital control systems or

mathematically oriented courses at the graduate level.

The z-transform is to discrete-time systems and difference equations

what Laplace transforms are to continuous-time systems and differential

equations. Many of the same properties apply that we saw in discussing

Laplace transforms. First we will start with the definition of the z-transform

of a discrete-time signal.

The definition of a z-transform of a discrete-time signal x[n] is

X.z. .

XN

n. N

x.n z n (2.45)

where z is a complex number.

The inverse z-transform is given by

x.n  . 1

2pj

I

C

X.z.zn 1dz (2.46)

where C is a counterclockwise path that encloses the origin and the region

of convergence.

Region of convergence is defined as the set of points in the complex z

plane for which the z-transform in (2.45) converges. Notice that (2.45) is an

infinite sum, so it will only give a finite value under certain conditions for a

given discrete-time signal x[z]. For the z-transform to be complete, the

region of convergence must be specified along with X(z).

In practice, as with Laplace transforms, it is more common to use tables

than the definitions in (2.45) and (2.46). However, the definition of the

z-transform is a bit easier to use than the Laplace definition because one can

consider multiplication by z-n to be a delay operator, and difference

equations used to describe a discrete-time system can often be expressed as a

linear combination of delays.

Let us look at an example of a signal and its z-transform to demonstrate

these ideas. Suppose x[n] is given by

x.n  .

e n; n   0

0; n < 0

(2.47)

Using (2.45), the z-transform is

X.z. .

XN

n.0

e nz n (2.48)

Note that the lower limit of the summation is zero because x[n] . 0

when n is less than 0. Simplifying the expression gives

X.z. .

PN

n.0

.ez. n

.

PN

n.0

.ez. 1

n

. 1

1   .ez. 1

(2.49)

The last step was obtained using the relationship

P

Nn

.0 xn . 1

1   x. This

relationship only holds if jxj is less than 1; otherwise, the series does not

converge. This restriction leads us to the region of convergence (ROC). In

the above example, the ROC is

.ez. 1

< 1 (2.50)

or

jzj > e 1 (2.51)

Because z is a complex number, this last inequality says that z must lie

outside a circle of radius e 1 for the series to converge. Graphically, the

region of convergence is the area shown in Figure 2.32.

Thus the complete z-transform is

X.z. . 1

1   .ez. 1; jzj > e 1 (2.52)

As with Laplace transforms, we introduce notation and properties that

will be useful to us. First is to define the notation for signals and their

transforms. These time domain signals are represented by lower case letters

and are shown explicitly to be functions of n. Their z-transforms are

represented by the same letter capitalized and are functions of z.

Table 2.3 Notation for z-Transforms

Time Domain

Signal z-Transform

x.n 4X.z.

Im(z)

Re(z)

e–1

Figure 2.32 The region of convergence for z shown on the complex plane. The

z-transform converges for values of z in the shaded region.

Table 2.4 Important Properties of z-Transforms

Property

Time Domain

Signal z-Transform

Linearity ax1.n  . bx2.n 4aX1.z. . bX2.z.

Time shift x.n   n0 4z n0X.z.

Some properties of z-transforms will be particularly useful to us as we

derive transfer functions: linearity and time shift. The notation is illustrated

in Table 2.3. These properties are illustrated in Table 2.4.

As a simple first example, consider the difference equation in (2.53), and

let’s work through the process of generating its transfer function.

x.n  . 3x.n   1  . 4x.n   2  . y.n    y.n   2  (2.53)

becomes

X.z. . 3z 1X.z. . 4z 2X.z. . Y.z.   z 2Y.z. (2.54)

and the transfer function Y.z.

X.z. is

Y.z.

X.z.

. 1 . 3z 1 . 4z 2

1   z 2 (2.55)

or equivalently

Y.z.

X.z.

. z2 . 3z . 4

z2   1

(2.56)

These two forms of transfer function are used in practice, depending on

the field of study. In DSP, it is common to express transfer functions for

discrete-time systems in terms of negative powers of z as in (2.55) because

of their association with delay operations. For control systems, positive

powers of z are typically used. As we will see in the MATLAB example

later in this section, either convention may be used as long as the user

specifies which he or she is following.

The most common discrete-time system that we will see in studying

mechanical systems comes about through sampling a process for computer

control. As an example, we will work through a thermostat system in detail

and see the effects of this sampling on the model.

Consider a computer-controlled heating system represented by the

block diagram in Figure 2.33. In this situation, the computer samples the

room temperature at discrete intervals using an analog-to-digital converter

(ADC). Note that the room temperature is denoted by TI(t) because it is a

continuous-time signal. The sampled version of the room temperature

coming out of the ADC is TI[n], a discrete-time signal. Similarly,

the computer outputs a discrete-time signal qh[n], which goes through

a digital-to-analog converter (DAC) to produce a continuous-time

signal qh(t).

For our modeling purposes, we assume the ADC has infinite resolution.

It simply samples the continuous-time signal and outputs the exact value of

the signal at integer multiples of the sampling time ts. The value of the input

signal is captured at each sampling instant so that the relationship between

the two signals is

TI .n  . TI.nts. (2.57)

This relationship between the signals is shown graphically in

Figure 2.34. Figure 2.35 shows how this relationship can be modeled in a

block diagram as a switch that closes momentarily every ts seconds. Note

that this model is an idealized version of how the ADC behaves, and it

ignores quantization error. In an actual ADC, the sampled value is

represented by a finite number of bits. For example, in 12-bit sampling, the

value of the signal is quantized into one of 212 . 4096 values.

The DAC is modeled differently from an ADC. Typically a DAC is

represented using what is called a zero-order hold (ZOH) equivalent

model. In this case, the input to a DAC is a discrete-time signal, and the

output is the input value held constant for the time between samples. The

result of the conversion is a piecewise constant signal as shown in

Figure 2.36.

If we consider the input and output to be continuous-time signals, then

we can derive the transfer function of the DAC in terms of the Laplace

transforms of the input and output signals. For example, if the input is a unit

impulse (typically denoted by d(t)), the output is a rectangular pulse whose

width is ts, which can be expressed as the difference between a unit

step (denoted by u(t)) and a delayed unit step (denoted by u(t   ts)).

See Figure 2.37. Using the fact that the Laplace transforms of d(t), u(t), and

u(t   ts) are 1, 1s

, and 1s

e tss respectively, the transfer function of the DAC is

Y.s.

X.s.

. 1

s

.1   e tss. (2.58)

Often it is convenient to combine the DAC with the continuous-time

plant that follows it into a single transfer function using the z-transform.

We will do this in the context of the temperature control system introduced

in this section. First, we derive the transfer function for the

continuous element in the system. Consider a room as shown in

Figure 2.38 that has perfect thermal insulation everywhere except for one

wall.6 We can derive the room’s dynamical model using the governing

relationships for heat flow, temperature, and temperature difference. The

first relationship is

q . 1

R

.T1   T2. (2.59)

where q is the heat flow through the material, R is its thermal resistance,

and T1 and T2 are the temperatures on either side of the material. The second

relationship is

T_ . 1

C

q (2.60)

where T is the temperature of an object, C is its thermal capacity, and q is

the total heat flow into the object.

Applying (2.59) and (2.60) to the room, we get

T_ I . 1

C

.q1 . qh. (2.61)

and

q1 . 1

R

.TO   TI . (2.62)

where TI is the room temperature, TO is the outside temperature, C is the

thermal capacity of the room, R is the thermal resistance of the uninsulated

wall, q1 is the heat flow into the room through the uninsulated wall, and

qh is the heat source in the room.

Substituting (2.62) into (2.61), the model for the room is

T_ I . 1

C

1

R

.TO   TI. . qh

(2.63)

Simplifying the model to contain only one variable, let Td denote the

difference between the inside and outside temperatures.

Td . TI   TO (2.64)

Then

T_ d . T_ I   T_ O

. T_ I

(2.65)

The last simplification can be made if we assume the outside temperature

is slowly time varying compared with the inside temperature

(T_ O z 0). Substituting (2.64) and (2.65) into (2.63) gives a modified

model of the room.

T_ d .  Td

RC

. qh

C

(2.66)

Taking the Laplace transform (because we are still dealing with a

continuous-time system at this point) and rearranging gives the transfer

function for the room, the continuous plant in the heating system. Denote

this transfer function by Gp(s).

Gp.s. . Td.s.

Qh.s.

.

1C

s . 1

RC

(2.67)

Now let us combine this continuous plant with the DAC and ADC to

give us a ZOH model of the plant as shown in Figure 2.39.

Working through the block diagram in Figure 2.39, we will derive the

transfer function for the discrete-time system from qh[n] to Td[n]. Let the

input be an impulse.

qn.n  . d.n  (2.68)

Then taking the z-transform gives

Qh.z. . 1 (2.69)

Given the impulse input, the output of the DAC is

qh.t. . u.t.   u.t   ts. (2.70)

where

u.t. .

1; t > 0

0; t < 0

(2.71)

With u(t)   u(t   ts) as the input to the plant, the output is the step

response given by Td(t) . w(t)   w(t   ts), where

w.t. . L  1

Gp.s.

s

(2.72)

and

w.t   ts. . L  1

e tssGp.s.

s

(2.73)

In these expressions, L -1 denotes the inverse Laplace transform. Then

the output of the ADC is

Td.nts. . w.nts.   w.nts   ts.

. w.nts.   w..n   1.ts.

. w.n    w.n   1

(2.74)

and the z-transform of the output is

Td.z. . W.z.   z 1W.z.

. .1   z 1.W.z.

. .1   z 1.Zfw.n g

. .1   z 1.Zfw.nts.g

. .1   z 1.Z

L  1

Gp.s.

s

(2.75)

In these expressions, Z{$} denotes the z-transform. The transfer function

of the DAC, continuous plant, ADC series combination Geq(z) is then

Geq.z. . Td.z.

Qh.z.

. .1   z 1.Z

L  1

Gp.s.

s

(2.76)

We will now substitute the room model into (2.76).

Gp.s.

s

.

1C

s

s . 1

RC

(2.77)

Then taking the inverse Laplace transform (obtained from a table of

Laplace transforms)

L  1

Gp.s.

s

. R

1   e t=RC

(2.78)

and now taking the z-transform of the sampled version of this signal gives

Z

L  1

Gp.s.

s

. R

z

z   1

z

z   e ts=RC

. R

1   e ts=RC

z

.z   1.

z   e ts=RC

. R

1   e ts=RC

.1   z 1.

z   e ts=RC

(2.79)

where this expression uses (2.49) and

Z.1. .

PN

n.0

z n

. 1

1   z 1

. z

z   1

(2.80)

Therefore, the ZOH equivalent transfer function becomes

Geq.z. . .1   z 1.R

1   e ts=RC

.1   z 1.

z   e ts=RC

. R

1   e ts=RC

z   e ts=RC

!

(2.81)

or in terms of negative powers of z,

Geq.z. . R

0

B@

1   e ts=RC

z 1

1   e ts=RCz 1

1

CA

(2.82)

It is worth noting here that an assumption has beenmade: the same sample

rate ts is used for both the analog-to-digital and digital-to-analog conversion so

that the whole system is in sync. It is important to know this sampling rate

because it greatly affects the systemperformance. Also, in this sampling process,

information between samples is lost and cannot be recovered. We will see the

effects of the sampling rate in the following MATLAB example.

2.3.3.1 MATLAB Example: Model of a Computer-Controlled

Heating System

We will now simulate the step response of the system in (2.81). But before

simulating in MATLAB, we need to establish numbers to use for R and C,

and this requires a few steps.

First, there are some values that we will assume for this situation: The

MATLAB code shown below provides the step response for both the

continuous-time and discrete-time transfer functions of the system for side

by side comparison.

% heating.m

close all

clear all

% Define constants for the system

ts . 10; % sampling interval in seconds

R . 0.0242; % Ks/J

C . 44100; % J/K

a . exp(-ts/(R*C));

% Transfer function of continuous system

num . 1/C;

den . [1, 1/(R*C)];

Gp . tf(num,den)

subplot(2,1,1)

step(Gp,7000)

title('Step Response of Continuous System')

% Transfer function of discrete system

num . R*(1-a);

den . [1, -a];

Geq . tf(num,den,ts)

subplot(2,1,2)

step(Geq,7000)

title(['Step Response of Discrete System with Sampling Time of

',num2str(ts),' seconds'])

In the first part of the code, the constants in the system R and C are

defined as those calculated in Table 2.5. The sampling time ts is initially set

to 10 s, and the variable a is defined to simplify entering the transfer function.

The next section of code defines the continuous transfer function from

(2.67). The method used here is exactly that which we have seen in Section

2.3.2.1.

The last section of code defines the discrete transfer function from (2.81).

Here we see the tf command being used similar to the continuous case. The

numerator and denominator of the transfer function must be defined by the

coefficients of z in decreasing powers. In this case, positive powers of z

are used, and this is the default setting for MATLAB. The only difference

between the definitions of the continuous-time and discrete-time systems is

that the user must specify the sampling time.

Geq . tf(num,den,ts)

The plot resulting from this code is shown in Figure 2.40.

What do these plots tell us? Remembering back to the transfer functions

in (2.67) and (2.76), they are defined as Td.s.

Qh.s. and Td.z.

Qh.z., respectively. That is,

temperature over heat in both cases. The step command assumes the input